Public Transport Capacity Analysis Procedures                  for Developing Cities

Case Study: Transmilenio, Bogota, Colombia

Ivan Sanchez and Jack Reilly,                                       Rensselaer Polytechnic Institute

May, 2011

Supported by the Transport Research Support Program, TRS of the World Bank with financing from the U.K.’s Department for International Development, DFID.

Table of Contents

1.	Introduction	3

1.1.	Description of Transmilenio	3

1.1.1.	Vehicles	4

1.1.2.	Running Ways	6

1.1.3.	Stations	8

1.1.4.	Fare Structure and Fare Collection	12

1.1.5.	Service Configuration	13

1.1.6.	Operation at Stops	13

2.	Bus Stop Capacity Assessment	15

2.1.	Determining the Critical Stop	15

2.2.	Understanding the Operating System at Stops	16

2.2.1.	Stop at Calle 100	16

2.2.2.	Stop at Calle 76	18

2.3.	Determining the Mean Dwell Time	18

2.3.1.	Clearance Time	19

2.3.2.	Separation Time	19

2.3.3.	Allowance for Arrival Randomness	20

2.3.4.	Traffic Signal Interference	21

2.4.	Vehicle Capacity	22

2.5.	Peak Hour Factor Calculation	23

2.6.	Platform and Station Access Capacity	24

2.6.1.	Platform Capacity	24

2.6.2.	Walkway Capacity	25

2.6.3.	Turnstile Capacity – Entrance	26

2.6.4.	Entrance Capacity – Ticket Vending Kiosk	26

2.6.5.	Turnstile Capacity – Exit	26

List of Figures

Figure 1 – Transmilenio System Map	6

Figure 2 – Transmilenio Bi-articulated Bus	7

Figure 3 – Transmilenio Articulated Bus	8

Figure 4 – Transmilenio Running Way – Urban Arterial	9

Figure 5 – Transmilenio Running Way - Expressway	10

Figure 6 – Access Ramp to Platform at Calle 100	11

Figure 7– Schematic View of Stops  – exiting picture is a placeholder	12

Figure 8 – Bus Stop at Calle 100	13

Figure 9 – Loading Platform	14

Figure 10 - Faregates	15

Figure 11 – Congestion at Stop at Calle 100	19

Figure 12 –Schematic Illustration of Calle 100	19

Figure 13 - Schematic Illustration of Calle 76	20

Figure 14 – Walkway to station platform	27

Figure 15 – Exit Faregates	28

List of Tables

Table 1 –Dwell Time Assessment –Stops at Calle 76 and 100	22

Table 2 – Variability of Dwell Time and Headway	23

Table 3- Running Way Capacity - Calle 76	24

Table 4- Running Way Capacity - Calle 100	24

Table 5 – Peak Hour Factor	26

Table 6 – Platform Capacity Estimate	27

Case Study

Transit Capacity Concepts for Developing Cities

Transmilenio Bus Rapid Transit, Bogota, Colombia

# Introduction

Two case studies were developed using the procedures described in the manual Public Transport Capacity Analysis Procedures for Developing Cities.  One case study was for a bus rapid transit system and the other for a rail rapid transit system. The bus case study was developed from data from Bogota, Colombia and the rail case study was developed from data from Medellin, Colombia.

The purpose of the case studies was to determine if the procedures in the manual were of sufficient detail to provide reasonable results with moderate analytical effort. In the case studies, agency electronic data, supplemented with some field data was used. The data from the case studies was also useful in determining default values for certain attributes of the transit system elements which can be used if no local data are easily available.  Examples of these are headway and service time variance, two variables required to estimate the throughput capacity of a transit line.

# Description of Transmilenio

Transmilenio, S.A. is the operator of the bus rapid transit network in Bogota.  Transmilenio is a city agency which contracts bus operations to a number of private contractors, but retains controls of fares and service levels. The system infrastructure is owned by the City of Bogota. The rolling stock is owned by the respective contractors.

By any measure, Transmilenio is an extremely huge operation.  The system consists of a network of routes (See Figure 1) using exclusive lanes and high capacity buses, primarily articulated coaches. It has all of the characteristics of an advanced bus rapid transit system including, among other things, off-board fare collection and high level platforms to enable level boarding and alighting of customers. Daily ridership on the network is about 1,600,000 and the maximum number of passengers per direction per hour is about 43,000 along Av. De Caracas, the largest capacity bus line in the world. This number surpasses the maximum volume of most rail rapid transit system in the world.

Calle 100

Calle 76

Figure 1 – Transmilenio System Map

# Vehicles

The Transmilenio fleet consists of 1,500 articulated buses each with a seating capacity of 48 and an overall capacity of 160 at a standee density of 7/square meter.  The buses have a diesel propulsion system.

The buses are high floor which enable level boarding and discharging at high level stations. The buses are of nearly uniform design despite their being manufactured by a variety of coach builders. Recently, Transmilenio took delivery of 10 bi-articulated buses with a capacity of 240 with the same implied maximum passenger density of 7 per square meter. These are deployed on a single route. More widespread deployment is constrained by a platform geometry (position of doors at platforms and the spacing between successive berths at a single stop) designed for 18m single articulated buses 18 meters long. Despite this restriction, another 240 bi-articulated buses are on order.

Illustrations of the buses are shown in Figure 2 and Figure 3        below.

Figure 2 – Transmilenio Bi-articulated 27 Meter Bus (Image Courtesy of Volvo)

Figure 3 – Transmilenio 18 Meter Single Articulated Bus

# Running Ways

There are two types of running way cross-sections in the Transmilenio network.  In the center city along arterial streets, there are two running lanes in each direction.  Along limited access highways, there is a single running lane with a passing lane at stations. These are illustrated below.

Figure 4 – Transmilenio Running Way – Urban Arterial

-Figure 5 – Transmilenio Running Way – Expressway

# Stations

The Transmilenio stations are all center platform, median stations. Along limited access highways, the stations are accessed by footbridges from either side of the roadway. Stations generally have a single point of entry for customers where customers can purchase tickets at a staffed kiosk. Platforms are about 5 meters wide. The floor elevation of the platform is about 0.9 m above the roadway surface enabling level access to the buses serving the station.

Figure 6 – Access Ramp to Platform at Calle 100

Each station has two or three “vagons” which are boarding areas within the stop.  A vagon may have one or two discrete boarding locations, followed by a queuing area for a single bus along the station platform.   Each route is assigned to a unique loading area. Buses bypass stations depending on their route designation.

Figure 7 –Schematic Views of Stops

The individual vagons can be randomly accessed by entering buses. However, a vagon with two boarding locations requires sequential access to the boarding areas. There is a glass screen along both sides of the station with sliding doors aligned with the doors on the buses. This not only enhances safety but improves capacity of the platforms since awaiting passengers can safely stand almost to the edge of the platform.

Figure 8 –Bus Stop at Calle 100

In addition to serving as a boarding location, the platforms act as a buffer space for waiting customers during peak periods when it is unlikely that passengers will be able to board the first arriving bus going to their destination due to either high levels of through passengers on arriving buses or high boarding levels at the station.  The largest stations may service up to 250 buses per hour in each direction.

Figure 9 –Loading Platform

# Fare Structure and Fare Collection

The Transmilenio fare is 1700 pesos per one way trip.   There are no zone or peak period surcharges.  All entering customers must use a wireless RFID card to enter through a turnstile.  The card is a stored value card which may be purchased from ticket agents at each stop.   There are also vending machines for these tickets located at places away from the stations.  There are no discounts for multiple rides. Exiting passengers must travel through a turnstile.  An illustration of the fare gates appear in Figure 10. Given the large passenger volumes, fare gates often have an attendant to assist customers unfamiliar with the fare collection system and to assist customers with an occasional defective fare card.

Figure 10 – Fare Gates

# Service Configuration

The service scheme for Transmilenio is fairly complicated. Routes are designed in such a way that passengers have an ability to reach most other stations from their boarding station without the need to transfer.  Some routes are limited stop, while others go non-stop virtually terminal to terminal.

The relatively complex service design reduces transfers between vehicles to complete journeys. Between vehicle transfers would further congest already congested stations. During peak hours, up to 15+ individual may serve a particular station, with have headways on the order of three or four minutes/each.

# Operation at Stops

Bus operations at stops are complicated by the geometry of the boarding locations. Since there are glass platform screens which open when bus doors are in proper alignment with the screen, buses must stop at prescribed locations on the platform.  A typical platform configuration for busy stops is a platform of about 125 meters total length, with a boarding berth, a queuing area for two buses, followed by two adjacent boarding berths and a queuing area for an additional two buses. (See Figure 7).  The modules which comprise the boarding locations, vagons. Stops can have two or three vagons. Each vagon can be accessed randomly by an arriving bus. Each bus route is assigned to a specific vagon and in the case of multiple berth vagons, a specific berth. Buses operate according to a first in-first out queue within each vagon.

Each stop has a bypass lane for buses not stopping at the respective stop. The geometry of the station is such that the second waiting bus at a vagon impedes the re-entry of buses in the upstream vagons to the traffic stream. These factors make a capacity assessment complex.

# Bus Stop Capacity Assessment

The procedures in the manual were used to estimate the capacity of the system at its critical links. The primary assessments were the determination of the maximum service frequency and the effective vehicle loading at the critical stop. Supplementary capacity assessments of other elements such as fare collection, station entrances and walkways were also undertaken.  The data sources for these studies were primarily electronic records of Transmilenio, supplemented by some field observation and interviews with Transmilenio staff.

The procedures in the analysis followed the procedures in table Chapter 3 of the manual. Since the stops were center median, there was no interference by the interaction of right hand turning traffic and pedestrian crossing. Further, this was not intended to recommend a set of actions to expand capacity.  Accordingly, an analysis of alternative measures to increase capacity was not undertaken.

The outline of the tasks is as follows:

- Determine critical stops in the network

- Understand the operations scheme at each stop

- Collect field data on dwell time means and dispersion (standard deviation)

- Collect field data on pattern of arrival (or make assumption based on default value)

- Using tables in Chapter 3, based on simulation, determine capacity of stop in vehicles per hour

- Assess capacity of vehicles

- Conduct other capacity assessments

  - Platforms

  - Walkways

  - Fare collection

  - Exit gates

# Determining the Critical Stop

Data from Transmilenio were reviewed to determine the critical stop.  While a metric of the mean dwell time plus two standard deviations about the mean is ideal, transit operators know, with reasonable precision, the stops which create the most serious operational challenges and why they do. Based on discussion with Transmilenio staff, two critical stops were identified, one in the morning and the other in the afternoon. In the morning, the stop at Calle 100 was selected because of the high passenger discharge load. The area surrounding Calle 100 is a high density commercial area. About 10,000 customers per hour discharge at that station in the afternoon. Further, it is also an important work destination, with large number of routes serving it.  This stop is along a limited access highway. As a result, there is no conflict between buses and general or crossing traffic. The critical stop in the afternoon was the stop at Calle 76 on Av. De Caracas. This stop also serves a high density commercial area.  However, since this stop is on an arterial street in the downtown area, there is interference from crossing traffic and the traffic signal system which impedes capacity. This stop is also near the maximum load segment of the network resulting in a high occupancy of buses arriving at the station, limiting their ability to accept boarding passengers. The locations of these stops are shown in the system map.

# Understanding the Operations at Stops

# Stop at Calle 100

At stop 100, each of the three berths was assigned to one of three route groups. The assignment of buses to route groups was done in such a way that (1) passenger loads are balanced across the three groups and (2) routes within a route group had common segments so that many passengers could board any of the arriving buses at the berth regardless of its route designation.  The berth configuration is a single berth followed by a space for queuing the next arriving bus followed by a pair of berths (berth 2 and 3) adjacent to each other with a queuing space behind the berth pair. Schematically, this is illustrated below.  This configuration enables buses in berth 1 and 2 to reenter the traffic stream being delayed solely by the arrival of through buses in the outside lane.

Figure 11 below illustrates congestion during the peak hour at Calle 100. In the observed case, instead of propagating, the queue was cleared in a matter of minutes.

Figure 11 – Congestion at Stop at Calle 100

Figure 12 –Schematic Illustration of Calle 100

# Stop at Calle 76

At the stop at Calle 76, each of the five berths was assigned to one of five route groups.  The berth configuration is a single berth followed by a space for queuing the next arriving bus followed by a pair of berths (berth 2 and 3) adjacent to each other with a queuing space behind the berth pair. There is a queuing space for one bus followed by Berths 4 and 5.  The assignment of berths to routes is shown below.

Operation of the stop is treated as a single berth stop with a queuing area followed by two two-berth stops each with a single bus queuing area.

Figure 13 -Schematic Illustration of Calle 76

# Determining the Mean Dwell Time

The dwell time of buses at stations is the key determinant of station capacity. In the analysis, each vagon was treated as a separate stop with one or two boarding locations at the stop.

Transmilenio has a very sophisticated vehicle location system which assists in service supervision and control. One of the data items collected by the system is the time between door opening and door closing of arriving buses. This data provides the basis for estimating the mean and distribution of dwell times. Once a bus comes to a stop at the boarding location, it takes about 1.5 seconds for the door to open and accept passengers. A similar delay occurred between the time the door starts to close and the bus started departure from the stop.  Accordingly, 3 seconds were added to the electronic data recording to estimate dwell time distribution.

The minimum time between successive buses at a berth includes three other time components.  These are (1) clearance or re-entry time, (2) minimum separation time and (3) an allowance for randomness in the headway gap between successive buses.

Table 1– Door Open and Close Time

| Stop | Time Period | Dwell Time Mean (sec.) | Standard Deviation | Coefficient of Variation |
| --- | --- | --- | --- | --- |
| Calle 100 | AM Peak | 24 | 17 | 0.71 |
|  | PM Peak | 22 | 14 | 0.64 |
| Calle 72 | AM Peak | 19 | 15 | 0.79 |
|  | PM Peak | 20 | 10 | 0.50 |

# Clearance Time

Buses at stops must re-enter the traffic stream. Insofar as the adjacent lane is exclusively for buses, there are likely to be frequent headway gaps of sufficient duration to allow bus reentry. The vehicle volume in the bypass lane includes buses which are bypassing the station and buses stopped at the station upstream from the re-entering bus. The maximum volume of such buses is about 300 vehicles per hour.  Using data from the TCQSM, the re-entry time would be about 3 seconds.

# Separation Time

If an arriving bus is queued behind a bus at the loading berth, there is a time between the movement of the departing bus and the stopping of the entering bus.  This allows for the departing bus to clear the platform and allow for a safe stopping distance for the entering bus. Based on several observations this time is about 12-13 seconds. This is consistent with the mechanics of bus operations. The separation time includes reaction time for the following driver to realize that the leading bus is moving, acceleration of 1 meter per second per second for about 5 seconds and deceleration at the same rate. These acceleration and deceleration rates are comfortable levels for passengers and within the propulsion capability of the buses.

The total mean service time which includes dwell, separation and re-entry times for each of the critical stops is shown in Table 1 below.

Table 2 – Service Time Assessment –Stops at Calle 76 and 100

| Headway Component | Data Source | Stop 100 Time (sec.) | Stop 76 Time (sec.) |
| --- | --- | --- | --- |
| Station dwell time |  |  |  |
| door opening and closing time | Based on field observation | 3.0 | 3.0 |
| time doors are open (mean) | Based on data from Transmilenio | 24.0 | 20.0 |
| Safe separation time - Time between departing bus moving forward and arriving bus stopping at station | Based on field observation | 12.0 | 12.0 |
| Clearance (re-entry) time | Use TCQSM for volume = 300 buses per hour | 3.0 | 3.0 |
| Minimum time between buses | Sum of all elements | 42.0 | 38.0 |

# Allowance for Arrival Randomness

If all dwell times were uniform and the inter-arrival rate between buses were constant, the capacity calculation in buses per hour for a single berth would simply be a matter of dividing 3,600 (the number of seconds per hour) by the minimum service time between successive buses. However, randomness in these two characteristics requires that additional time be allocated to avoid bus-to-bus conflict. The higher this buffer time, the lower the probability that arriving buses will be blocked from entering the station.

In the case of the two critical stops, there is concern about buses not being able to enter the queuing space associated with the vagons, since this will cause waiting buses to occupy the bypass lane, impeding the flow of buses which bypass the stop. Further at very high failure rates, there is a concern that delays to through buses will propagate and cause growing queues of arriving buses. For the purpose of this discussion a very low failure rate of 10% was assumed for buses which might impede through buses.

Using data captured electronically by Transmilenio, the following estimates of coefficient of variation of headway and dwell time were determined (Table 2).

Table 2 -Variability of Service Time and Headway

|  | Mean (sec.) | Standard Deviation (sec.) | Coefficient of Variation |
| --- | --- | --- | --- |
| Headway | 55 | 51 | 0.93 |
| Service time | 40 | 12 | 0.30 |

The tables in Chapter 3 for multiple berth stops were used to determine the capacity of each of the vagons. In using the tables, the high level of headway coefficient of variation (0.8) and the low level of headway coefficient of variation (0.4) was assumed. In one of the key stops (Calle 76), there is a traffic signal which impedes the free flow of buses at the station.  This is discussed below.

# Traffic Signal Interference

At Calle 100, the stop on the arterial, there is no traffic signal interference. On the other hand, Calle 76, the downtown stop, is located at a major cross street with a traffic signal at either end.  At the south end of the stop, there is a space approximately one-bus long beyond the first loading area. This should result in minimal interference. At the north end of the platform, there is no such space and the green to cycle time is approximately .5.  This traffic signal affects only the capacity of the first boarding area (vagon). The traffic signal does not materially affect the flow rate of the through buses. The total bus flow rate is on the order of 250-300 per hour. A signalized intersection with 50% green time would have a capacity of about 400 vehicles per lane per hour in the passing lane.

The maximum capacity of the stations in vehicles per hour is taken from Tables 3-22 through 3-29 in the manual and summarized in Table 3  and Table 4 below. In each of these cases, a service time of 40 seconds was assumed.

Excerpts from the appropriate tables are replicated at the end of this case study as tables A-1, A-2 and A-3 Table 3 denotes which of the appendix tables was used for the calculation.

Table 3- Running Way Capacity - Calle 76

|  | Berths | Appendix Table | Capacity 5% Failure | Capacity 10% Failure | Capacity 25% Failure |
| --- | --- | --- | --- | --- | --- |
| Northbound |  |  |  |  |  |
| Vagon 1 (g/c=.5) | 1 | A-1 | 23 | 30 | 49 |
| Vagon 2 | 2 | A-3 | 48 | 62 | 76 |
| Vagon 3 | 2 | A-3 | 48 | 62 | 76 |
| Total capacity |  |  | 119 | 154 | 201 |
| Southbound |  |  |  |  |  |
| Vagon 1 (g/c=1.0) | 1 | A-2 | 23 | 35 | 54 |
| Vagon 2 | 2 | A-3 | 48 | 62 | 76 |
| Vagon 3 | 2 | A-3 | 48 | 62 | 76 |
| Total capacity |  |  | 119 | 159 | 206 |

Table 4- Running Way Capacity - Calle 100

|  | Berths | Appendix Table | Capacity 5% Failure | Capacity 10% Failure | Capacity 25% Failure |
| --- | --- | --- | --- | --- | --- |
| Northbound |  |  |  |  |  |
| Vagon 1 | 1 | A-2 | 23 | 35 | 54 |
| Vagon 2 | 2 | A-3 | 48 | 62 | 76 |
| Total capacity |  |  | 71 | 97 | 130 |
| Southbound |  |  |  |  |  |
| Vagon 1 | 1 | A-3 | 23 | 35 | 54 |
| Vagon 2 | 2 | A-3 | 48 | 62 | 76 |
| Total capacity |  |  | 71 | 97 | 130 |

# Vehicle Capacity

The second part of the system capacity is the estimated number of passengers on board each bus. Classical capacity analysis assumes that sufficient – are assigned to each route so that a passenger has a reasonable expectation of being able to board the next arriving vehicle going to his or her destination. In the case of Transmilenio, however, particularly at the busy stops such as those being included in this case study, the system is fully saturated. Nearly all buses are departing from these stops are at or above the stated capacity – 160 passengers per bus. Randomness in the arrival rate of customers at the stops and the departure rate of passengers on buses is absorbed by variation of the station occupancy. Accordingly, it is necessary to make sure that station capacity and the access to the station platforms such as turnstiles and entrance walkways be scaled in such a way to assure safe transportation of passengers.

From visual observation, buses are loaded in the 6 to 8 passengers per square meter range, with the higher values in the peak point of the peak period. The acceptable load standard of boarding customers is determined at least partially by the duration of time that a customer has waited for the bus. This depends on the service frequency and the number of through passengers on the arriving buses.

For the purpose of this calculation, it is assumed that within the peak of the peak time, the number of customers departing Stop 76 per bus was 160, the crush load standard.  A separate assessment of the peak hour factor appears in the discussion below.

# Peak Hour Factor Calculation

The peak hour factor is a measurement to downward adjust capacity estimates to account for the fact that ridership is not uniform over the peak period.

Using data from the Transmilenio fare collection system, the peak hour factor for the two busy bus stop was estimated. Table 5 below shows the number of passenger boardings at the two critical stations in 15 minute intervals throughout the two peak periods. The peak hour factor calculation is shown in the table as well.  The peak hour factors are extremely high, indicating that there is little peaking within the peak period. This also suggests that a number of customers are taking their trips at times to avoid the peak hour.

Table 5 – AM Peak Hour Factor

|  | Calle 100 | Calle 72 |
| --- | --- | --- |
| Start of Time Period | Monthly Boardings | Monthly Boardings |
| 7:00 AM | 36,852 | 40,856 |
| 7:15 AM | 41,418 | 42,556 |
| 7:30 AM | 48,480 | 45,360 |
| 7:45 AM | 51,813 | 52,650 |
| 8:00 AM | 48,676 | 51,158 |
| 8:15 AM | 37,262 | 40,382 |
| Peak hour | 190,387 | 191,724 |
| Peak 15 minutes | 51,813 | 51,813 |
| AM Peak hour factor | 0.92 | 0.93 |

Table 6 – PM Peak Hour Factor

|  | Calle 100 | Calle 72 |
| --- | --- | --- |
| Start of Time Period | Monthly Boardings | Monthly Boardings |
| 4:30 PM | 22,857 | 22,481 |
| 4:45 PM | 22,597 | 23,881 |
| 5:00 PM | 24,085 | 28,297 |
| 5:15 PM | 24,071 | 35,254 |
| 5:30 PM | 23,099 | 38,150 |
| 5:45 PM | 21,777 | 40,343 |
| 6:00 PM | 20,945 | 41,020 |
| 6:15 PM | 20,634 | 38,895 |
| Peak hour | 93,852 | 158,408 |
| Peak 15 minutes | 24,085 | 41,020 |
| PM Peak hour factor | 0.97 | 0.97 |

The high peak hour factor suggests that some passengers are shifting their home or work departure time to avoid the congestion during the peak hour.

# Overall throughput Capacity

The overall throughput capacity of the two critical stations can be computed from the previous assessment. We compute the capacity as the number of persons who board at the critical station plus the number of passengers on buses which stop at the critical station. Passengers in buses which bypass the station are not included in this total.  The capacity of the bypass lane is much larger than the boarding lane and is determined by operations at the critical traffic intersection – the one with the shortest ratio of green time to cycle time (g/c) in the direction of traffic of the buses.  At Calle 76, the g/c ratio is about 0.5.  The saturation flow rate of the bypass lane with articulated buses is about 800. That is, absent traffic signals, about 800 buses could flow along the busway. The traffic light reduces this maximum flow rate in half for an effective capacity of about 400. The actual bus volume including those which stop at the station and those which bypass the station, even during the busiest period is on the order of 250-300 buses per hour.

Table 7 below shows the assessment of line capacity for buses which stop at the respective morning and afternoon busiest station. The passenger loads on through buses which bypass these stations are not included in these totals.

Table 7 – Throughput Capacity

|  |  |  | Platform Capacity (buses/hr.) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station | Period | PHF | 5% Failure Rate | 10% Failure Rate | 25% Failure Rate | Vehicle Capacity | Line Capacity at 10% Failure Rate | Line Capacity at 25% Failure Rate |
| Calle 100 | AM | 0.92 | 71 | 97 | 130 | 160 | 14,300 | 19,100 |
| Calle 76 | PM | 0.97 | 119 | 154 | 201 | 160 | 23,900 | 31,200 |

# Platform and Station Access Capacity

# Platform Capacity

Since the two stations being reviewed are at capacity, the role of station capacity which serves as a buffer between the accumulated arriving passengers and cumulative departing passengers is quite important. The typical sizing of station platforms for rail systems with a single rail line is to estimate the arrival rate of customers and assume that the service operates in an erratic fashion with the design headway for station capacity being twice the published headway.

At the two stations under review, there is a high probability that customers on the platform will not be able to board the next arriving bus with the desired route designation since arriving buses may be already crowded or there is a high boarding volume at the station.  Under these circumstances, a design criterion would be to include sufficient platform capacity to assure that customers can wait for a specific number of arriving buses before boarding. For example, if the design criterion was that customers would wait on average for the arrival of the second bus within a route, the estimated required capacity would be:

| C | = | 1.5 | * | Average | * | entering | / | allowable |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | route headway |  | flow rate |  | occupancy density |

In the case of the stops under consideration, the peak 15 minute entrance rate was 73 passengers per minute at Calle 100 and 124 per minute at Calle 72. The average route headway was 4 minutes. An allowable occupancy density would allow for passenger circulation between the loading berths. A loading standard of 4 persons per square meter would accomplish this. The estimated platform capacity requirement in these cases is shown in Table 8 below.

Table 8 – Platform Capacity Estimate

| Average route headway | 4 | Minutes |
| --- | --- | --- |
| Entering flow | 124 | Passengers per minute |
| Allowable density | 4 | Passengers per square meter |
| Required capacity | 190 | Square meters |

# Walkway Capacity

At the Calle 100 station, there is a walkway connecting the overhead pedestrian bridge to the station entrance. During the busy period, the maximum exiting flow rate was about 60 passengers per minute in a single line with a 6% grade. The maximum entrance flow rate was about 73 per minute.

The walkway also served as a queuing area for passengers waiting to enter the station.  The length of the walkway is about 80 m and is effectively 1m wide in the major direction. At about 4 passengers per square meter (a density that allows for pedestrian flowing), the queuing space allows for 320 passengers.

Figure 14 – Walkway to station platform

# Turnstile Capacity – Entrance

The turnstile entrances permit the flow of 25 passengers per minute per turnstile. The station entrances have 4 reversible flow turnstiles. During peak hours, three are in the major direction of travel and one is in the minor direction of travel yielding an entrance capacity of about 75 per minute. With an arrival rate of about 73 per minute, the entrance turnstile system at that station is operating at close to peak capacity.

Flow rates are enhanced by the presence of staffed attendants at the turnstiles. They provide assistance to passengers unfamiliar with the fare collection system and to assist in occasional malfunctions of the payment cards.

# Turnstile Capacity – Exit

Exiting customers must go through a turnstile. Passengers do not use their card on station exit. Observed peak flow rates were about 45 customers per lane per minute. This is close to the practical capacity of the exit.

Figure 15 – Exit Fare Gates

Table A-1 – Approximate Capacity of Single Berth, with Queuing Area

Green/cycle time =0.5

(Vehicles per hour)

|  |  |  | Failure Rate |  |  |
| --- | --- | --- | --- | --- | --- |
| Service Time (sec.) | Service Time CV* | Headway CV | 5% | 10% | 25% |
| 40 | 40% | 40% | 43 | 46 | 54 |
|  | 40% | 80% | 23 | 30 | 49 |
|  | 80% | 40% | 32 | 41 | 46 |
|  | 80% | 80% | 17 | 27 | 40 |

* CV – coefficient of variation = standard deviation/mean

Table A-2 – Approximate Capacity of Single Berth, with Queuing Area

Green/cycle time =1.0

(Vehicles per hour)

|  |  |  | Failure Rate |  |  |
| --- | --- | --- | --- | --- | --- |
| Service Time (sec.) | Service Time CV* | Headway CV | 5% | 10% | 25% |
| 40 | 40% | 40% | 44 | 47 | 57 |
|  | 40% | 80% | 23 | 35 | 54 |
|  | 80% | 40% | 35 | 42 | 52 |
|  | 80% | 80% | 24 | 30 | 44 |

Table A-3 – Approximate Capacity of Double Berth, With Queuing Area

Green/cycle time =1.0

(Vehicles per hour)

|  |  |  | Failure Rate |  |  |
| --- | --- | --- | --- | --- | --- |
| Service Time (sec.) | Service Time CV | Headway CV | 5% | 10% | 25% |
| 40 | 40% | 40% | 55 | 67 | 78 |
|  | 40% | 80% | 48 | 62 | 76 |
|  | 80% | 40% | 46 | 51 | 61 |
|  | 80% | 80% | 39 | 44 | 66 |

Transmilenio has an electronic system which captures the door opening and closing times of buses arriving at stops as well as boardings and alightings by stop. Some analytical assessment of this data could have produced more analytically robust, but probably not a more accurate determination of the critical stop.